zawartość azotu mineralnego w glebie zanieczyszczonej substancjami ropopochodnymi*

Mirosław Wyszkowski, Agnieszka Ziółkowska

- Katedra Chemii Środowiska

Uniwersytet Warmińsko-Mazurski w Olsztynie

Słowa kluczowe: benzyna, olej napędowy, gleba, kompost, bentonit, wapno, N-NH4+,      N-NO3-

Streszczenie

Celem przeprowadzonych badań było celem było określenie oddziaływania zanieczyszczenia gleby benzyną i olejem napędowym w dawkach 0; 2,5; 5 i 10  cm3· kg-1 na zawartość N-NH4+ i N-NO3- w glebie. Do neutralizacji prawdopodobnego ujemnego wpływu tych substancji zastosowano kompost, bentonit i wapno. Olej napędowy działał zdecydowanie silniej na zawartość azotanów(V) niż benzyna, przy czym istotne zwiększenie zawartości N-NO3- w glebie wykazano do dawki 5 cm3 · kg-1, podczas gdy benzyna wpływała w analogiczny sposób przy najmniejszej dawce (2,5 cm3 · kg-1). Wyższe dawki substancji ropopochodnych nie wywoływały istotnych zmian (benzyna) lub działały ujemnie (olej napędowy) na zawartość azotanów(V) w glebie. Bentonit i wapno oraz kompost (tylko w obiektach z olejem napędowym) spowodowały istotne zmniejszenie zawartości N-NH4+ w glebie, należy przy tym zaznaczyć, że działanie to było większe w wazonach z olejem napędowym niż w obiektach z benzyną. Wszystkie zastosowane dodatki sprzyjały nagromadzaniu się N-NO3- w glebie w obiektach z benzyną, a bentonit i wapno w wazonach z olejem napędowym. Aplikacja substancji ropopochodnych oraz kompostu, bentonitu i wapna zwiększyła udział N-NO3- w N-mineralnym gleby. Zawartość mineralnych form azotu wykazywała uzależnienie od niektórych innych właściwości gleby.

the mineral nitrogen content in THE SOIL CONTAMINATED WITH PETROLEUM substances

Mirosław Wyszkowski, Agnieszka Ziółkowska

- Department of Environmental Chemistry

University of Warmia and Mazury in Olsztyn

Key words: petrol, diesel oil, soil, compost, bentonite, lime, N-NH4+, N-NO3-

Summary

The aim of the present study was to determine the effect of soil contamination with petrol and diesel oil (0, 2.5, 5 and 10 cm3 · kg-1) on the concentrations of N-NH4+ and N-NO3- in the soil.  The expected negative impact of these substances was neutralized with compost, bentonite and lime.  Diesel oil had a much stronger influence on the levels of nitrates (V) than petrol.  A significant increase in the N-NO3- content of the soil was observed up to a dose of 5 cm3 · kg-1, whereas petrol produced the same effect when applied at the lowest dose (2.5 cm3 · kg-1).  Higher doses of petroleum substances did not cause significant changes (petrol) or had a negative effect (diesel oil) on the nitrate (V) concentrations in the soil.  Bentonite, lime and compost (only in the treatments with diesel oil) considerably decreased the N-NH4+ content of the soil, but it should be stressed that their impact was stronger in the treatments with diesel oil than in those with petrol.  All additives contributed to the accumulation of N-NO3- in the soil in the treatments with petrol, while bentonite and lime - in those with diesel oil. The application of petroleum substances, compost, bentonite and lime increased N-NO3- proportion in N-mineral of soil. The concentrations of mineral nitrogen was dependent on some other soil properties.