CHARAKTERYSTYKA ZAWARTOŚCI OŁOWIU W GLEBACH PRZY DRODZE OLSZTYN-OLSZTYNEK

Zdzisław Ciećko, Andrzej Żołnowski, Mirosław Wyszkowski*

* Katedra Chemii Środowiska  Akademii Rolniczo-Technicznej im. M.Oczapowskiego;

10-718 Olsztyn, Plac Łódzki 4

Streszczenie

Celem badań było określenie oddziaływania spalin pojazdów samochodowych na zawartość ołowiu w glebie przy drodze nr 51 na odcinku Olsztyn-Olsztynek. Badania odniesiono do trzech różnych gleb występujących przy tej trasie. Była to gleba organiczna w Kortowie, gleba ciężka w Tomaszkowie i gleba lekka w Dorotowie. W każdej z gleb wyznaczono punkty pobrania próbek w odległości od osi jezdni - 10, 50, 90, 130 i 170 m. W każdym z punktów pobrano próby gleby z dwóch poziomów: 0-25 i 26-50 cm. Najwyższą koncentrację  ołowiu ogólnego stwierdzono w glebie organicznej, a najniższą w glebie lekkiej. We wszystkich glebach wyższą zawartość ołowiu odnotowano w warstwie wierzchniej niż w poziomie 26-50 cm. Najniższą kwasowość hydrolityczną i  najwyższy odczyn, zarówno w wodzie, jak i w KCl, we wszystkich glebach wykazano w punkcie  znajdującym się w odległości 10 m od jezdni.

Characteristics of lead content in soils near the road Olsztyn - Olsztynek

### Zdzisław Ciećko, Andrzej Żołnowski, Mirosław Wyszkowski

Summary

The aim of the study was to determine the effects of fumes from car engines  on lead content in soils close to road No 51 between Olsztyn and Olsztynek.  Studies have been made on three different soils localized near that road.  There were organic soil in Kortowo, heavy soil in Tomaszkowo and light soil  in Dorotowo. In each case sampling points of following distance from the  edge of the road were established: - 10, 50, 90, 130 and 170 m. From each  soil, the sample were taken from two layers at: the depth of 0-25 and 26-50 cm. The highest content of total lead was found in organic soil and the  lowest in light soil. In each samples higher lead content was found in upper  soil layer comparing to the depth of 26-50 cm. The lowest hydrolytical  acidity and the highest pH values measured in water and in KCl for each  studied soil were noted at the distance of 10 m from the road.